North America EMV Release Notes

3.1.2xx Series Contactless Only

Reader C150S, C150SE                                                                                              “x” indicates product(s) affected.

12/05/2025

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| Dev - EMV 3.1: New C150S WayneTap FW to for new Unified Package of Contactless Kernels CLK 3.0 - C150S Dev: 26100A02_25091802 - C150S Prod: 26100A02_25091801 Note that CLK3.0 CTLS FW packages are available in TeamCity and Jira Ticket; however, they are not part of default build as majority of customers are still on CLK1.0 | X | X | X |
| Dev - EMV 3.1: New C150SE (New Kernel) WayneTap FW to for new Unified Package of Contactless Kernels CLK3.0 - C150SE Dev: 73601000_25061832 - C150SE Prod: 73601000_25061831 Note that CLK3.0 CTLS FW packages are available in TeamCity and Jira Ticket; however, they are not part of default build as majority of customers are still on CLK1.0 | X | X | X |
| Dev - EMV 3.1: New C150SE (New Kernel) WayneTap FW to support MasterCard Conexxus Fleet CLK3.0 - C150SE Dev: 73601000_250092432 - C150SE Prod: 73601000_250092431 - C150S - not in scope (C150S will not support MC Conexxus Fleet) Note that CLK3.0 CTLS FW packages are available in TeamCity and Jira Ticket; however, they are not part of default build as majority of customers are still on CLK1.0 | X | X | X |

5/12/2025

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| Dev - EMV 3.1: New C150S\C150SE WayneTap FW (Dev and Prod) to fix EMV Contactless Mastercard COM01T01S02 Test Case - C150S - both dev and prod - 26100A02_25032702 - dev - 26100A02_25032701 - prod - C150SE (old kernel) - both dev and prod - 73601000_25040716 - dev - 73601000_25040715 - prod - C150SE (new kernel) - both dev and prod - 73601000_25040802 - dev 73601000_25040801 - prod | X | X | X |

11/5/2024

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| EMV 3.1: Amex US Common Debit - New CTLS Firmware to Support for "Preferred" Logic C150S: Production 26100A02 24070901 Development 26100A02 24070902 C150SE: Production 73601000 24091911 Development 73601000 24091912 | X | X | X |
| EMV 3.1: New CTLS Firmware to fix JCB L3 Test Case 213 05 Fails due to Incorrect POS Entry Mode Tag 9F39 Value (See CTLS firmware versions above) | X | X | X |

3/26/2024

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| EMV 3.1.212: New WayneTap Firmware to Retrieve Tag 9F0C for Discover (C150S\C150SE-OldKernel\C150SE-NewKernel) - Dev and Prod (CAB Files an d Firmware) - C150S Dev: 26100A02, Build Rev 24032602 - C150S Production: 26100A02, Build Rev 24032601 | X | X | X |

11/30/2023

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| EMV Contactless C150S Firmware to Return Tag 9F0C (Issuer Identification Number Extended) and the New LED Light Status for Firmware Upgrade 26100A02, Build Rev 231116 - C150S Dev: 26100A02, Build Rev 23111602 - C150S Production: 26100A02, Build Rev 23111601 | X | X | X |

06/01/2023

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| New C150S Firmware to fix Ghost Tap 26100A02, Build Rev 230224 - C150S Dev: 26100A02, Build Rev 230224A6 - C150S Production: 26100A02, Build Rev 230224A5 | X | X | X |

12/5/2021

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| Visa EMV Fleet 2.0 Implementation (Contactless) - This includes updated C150S firmware: - 26100A02, Build Rev 211101 - C150S Dev: 26100A02, Build Rev 21110104 - C150S Production: 26100A02, Build Rev 21110103 | X | X | X |

Reader C150SE

3/26/2024

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| EMV 3.1.212: New WayneTap Firmware to Retrieve Tag 9F0C for Discover (C150S\C150SE-OldKernel\C150SE-NewKernel) - Dev and Prod (CAB Files an d Firmware) - C150SE Dev: 73601000, Build Rev 23110902 (02 designates new kernel, dev) - C150SE Production: 73601000, Build Rev 23110901 (01 designates new kernel, prod) - C150SE Dev: 73601000, Build Rev 23110612 (12 designates old kernel, dev) - C150SE Production: 73601000, Build Rev 23110611 (11 designates old kernel, prod) | X | X | X |

10/7/2022

| Description | Anthem | T7/ECIM | Jade+SCD |
| --- | --- | --- | --- |
| Test C150SE Device/FW and Add FW to BitBucket 73601000, Build Rev 220706 - C150SE Dev: 73601000, Build Rev 22070604 - C150SE Production: 73601000, Build Rev 22070603 See table below for new kernel information | X | X | X |